20 years of evolution for the DORIS permanent network:

from its initial deployment to its renovation

Hervé FAGARD1

1 Institut Géographique National, Service de Géodésie et Nivellement, Saint-Mandé, France.

Fax : + 33 1 43 98 84 50, e-mail : herve.fagard@ign.fr

INTRODUCTION : HISTORICAL BACKGROUND

The realization of the DORIS system was decided jointly in the early eighties by the French space agency (CNES: Centre National d’Études Spatiales), the French national mapping agency (IGN: Institut Géographique National) and a research group in the field of space geodesy (GRGS: Groupe de Recherche en Géodésie Spatiale). Because of its experience in the field of the installation of geodetic networks, IGN has taken care of the deployment of the ground network and of the determination and publication of the stations coordinates.

An essential requirement for the precise computation of the orbits was to ensure an almost constant visibility of at least one ground station by the on-board receiver. On the other hand, to be able to express the orbit in a precise terrestrial reference system, the coordinates of a sufficient number of well distributed stations had to be available in the same system. To meet the orbit coverage requirement for the SPOT-2 satellite, it was estimated that the network should be made of approximately 50 stations, as evenly distributed as possible around the globe.

The very first DORIS station was Tristan da Cunha (code TRIA), which was installed by the Proudman Oceanographic Laboratory in June 1986. Then installation followed installation at a sustained pace, with about 10 new stations a year (figure ?) during the first two years. Then the deployment went on at a steady pace of about 5 new stations per year until the end of 1992.This date also marked approximately the end of the deployment of the first generation antennas which will be dealt with in Chapter ?. Figure ? shows the distribution of the 32 stations that made up the network on the official start of the DORIS system operation (end of January 1990), with visibility circles corresponding to the 12° cut-off angle used at that time.

Figure ?: map of the DORIS network upon SPOT-2 launch (January 1990)

As of 1993, the network deployment went on at a slower pace. The number of stations reached 49 – roughly the initial objective of 50 stations – by the end of 1993. A few new stations were added, some existing ones had to be moved to new locations following the closure of host agencies, or ???. All these new stations were equipped with the second generation antenna, and a few with the second generation beacon.

As of 2000, a general renovation was initiated, in order to improve the overall stability of the antennas reference point. Many stations were completely renovated or moved to a new location. A few new stations were installed, all meeting the new, more stringent requirements stability wise. The deployment of the third generation beacons started in 2002.

Figure ?: evolution of the DORIS network

# THE DEPLOYMENT OF THE EARLY NETWORK: THE ALCATEL ERA

Sites selection criteria.

The initial list of potential DORIS stations locations mainly ensued from the need of geocentric coordinates, the best source of which would be a co-location of the DORIS antenna with the space geodesy techniques available at that time: Very Long Baseline Interferometry (VLBI) and Satellite Laser Ranging (SLR). When none of these instruments were available, coordinates could be obtained through Doppler Transit or GPS positioning, either already determined in the frame of international measurement campaigns or to be measured by IGN at the same time as the DORIS equipment installation. This was notably the case at many islands, that were selected in order to meet the density and homogeneous distribution criteria for the network, even though no space geodesy measurements had ever been performed at these sites.

The concern for co-locations between the DORIS stations and tide gauges appeared later, with the growing interest for sea level rise related studies.

Selection of a host agency.

After a site had been a priori selected, a host agency had to be found, who would agree to host the station and take care of its maintenance, and where the following needs could be met :

- The transmitting beacon and its backup power supply needed to be in a room with moderate temperature and temperature gradient, with mains power supply available.

- The antenna had to be installed outside with a clear view above 10 degrees, on a structure that would allow to use the antenna supports – guyed tower or wall side mount – used at that time.

- It was necessary to check that the frequencies transmitted by DORIS would not be likely to interfere with existing receivers in the area, and the necessary frequency clearance had to be obtained.

- The host agency should agree to carry out some occasional maintenance operations at IGN’s request. This would include some minor verifications and adjustments, as well as sending out of order equipment for repair.

In order to check that the prospective host agency would match the above requirements, a questionnaire was sent which generally resulted in yes/no answers to a few questions, and a variable amount of details about the site layout.

Then it was necessary to obtain the prospective host agency’s agreement to host and maintain the DORIS station, which was in some cases materialized by a written agreement signed with IGN. Frequency clearance had also to be granted, which was generally handled by the host agency through an application with the relevant radio communications authorities. This negotiation stage generally took several months, sometimes more than one year.

Installation stage.

Once a host agency had been found and all the necessary authorizations granted, the installation was performed by IGN. This stage included :

- Dispatch and customs clearance of the equipment.

- Installation and starting up of the station.

- Training of the staff who would take care of the maintenance.

- Geodetic survey of the antenna’s reference point, and connection to another space geodesy technique, or to the local geodetic network.

In order to be able to adapt to the various site layout likely to be encountered, and for lack of detailed information allowing beforehand to determine exactly where and how the antenna and beacon would be installed, a standard set of antenna supporting devices was sent. This included several one-meter lattice tower sections, guy wires and a wall side mount for the antenna, and a home-made small rack for the beacon and batteries. The IGN technician who carried out the installation had to manage to find suitable locations for both the beacon and antenna, compatible with what was generally the most restrictive limitation of the DORIS equipment set: the very short – 10 m – cable length between the beacon and the antenna. In order to meet the good visibility requirement despite this limitation, many antennas had to be installed on building roofs or on top of two or three meter high towers, if not higher.

The antenna support layouts that resulted from such installations are dealt with in Chapter ??

# Description of the equipment

The first version of the equipment that made up a DORIS station consisted of:

- The beacon, version 1.0, manufactured by CEIS, France. This element (Figure ?), weighing 24 kg and designed to be integrated in a standard 19 inches rack, had to be installed inside a building with moderate temperature gradient. It could be programmed through an integrated man-machine interface consisting of a keyboard and a LCD screen. The beacon generates the DORIS signals: 401.25 MHz (6 W) and 2036.25 MHz (12 W).

- A box containing three 12V batteries, providing backup power to the beacon during power outages up to 72 hours.

- A weather station (Figure ?) measuring temperature, pressure and humidity. These parameters are transmitted through the 400 MHz modulated signal and can be used to correct for atmospheric propagation delays.

- A dual frequency and omni-directional antenna (Figure ?), manufactured by Alcatel. This antenna was bolted on an interface (consisting of a square horizontal plate welded to a vertical tube) which could be fastened to a variety of supports, in most cases a small lattice tower.

| Figure ?: DORIS beacon 1.0 (top) and Battery case (bottom) in a homemade rack | Figure ?: DORIS Alcatel antenna (left) on a one-meter tower and side wall mount. DORIS meteorological station on the right. |
| --- | --- |

# Alcatel antenna layouts

The most frequently used antenna support was a triangular, 17 cm sided lattice tower made of two or three one-meter sections, bolted together and set up on one of the following structures:

- A concrete pad on the ground, already available (figure )

- A concrete block specially built for the DORIS installation (figure )

- A building’s top terrace. (figure )

At a few sites where the antenna was installed on a roof, a clear sky view allowed to use only one tower section. Conversely, four sections had to be used at a couple of locations in order to avoid nearby signal obstructions.

| Figure ?: 2 m tower on a concrete pad (Goldstone: GOMA) | Figure ?: 2 m tower on a concrete block (Marion Island: MARA) | Figure ?: 3 m tower on the upper terrace of a building (Galapagos: GALA) |
| --- | --- | --- |

When such layouts were used, the tower itself was mounted on a square base plate which was bolted to the concrete support using expansion bolts. This base plate had a small vertical tube in its center, which prevented to see the ground mark if one had been set under the plate. In some cases the tube itself was used as the control mark instead. Such a control mark was destined to be used in the future to check the antenna stability on one hand, and as a marker of the antenna location in case of movement or accidental destruction of the antenna on the other hand.

Other designs have been used more rarely:

- Direct mount of the antenna interface on a roof, without using a tower (Figure ?)

- Steel pole instead of a tower (Figure ?)

- Tower mounted on a wall (Figure ?)

In some of these cases, no ground mark was present.

| Figure ?: Antenna interface mounted directly on a roof ; no tower (St Helena: HELA) | Figure ?: High steel pole, propped by very long guy-wires (Dakar: DAKA) | Figure ?: Side mount of a tower against a load-bearing pillar. No guy-wires… (Hartebeesthoek: HBKA) |
| --- | --- | --- |

Most towers were propped using stainless steel cable wires and turnbuckles, allowing a pretty strong and stable fastening of the tower. Nevertheless at a few sites the cable wires were very long or somewhat loose or even nonexistent, which would not guarantee a centimeter-level stability of the antenna. This was yet acceptable considering the expected positioning accuracy of the DORIS system at that time (10 cm).

By adjusting the tension of the stays, it was possible to center the antenna base (i.e. reference point) above the ground mark when present. However, none of the above antenna support designs allowed to precisely adjust the antenna verticality, i.e. to guarantee that the electrical phase centers – and notably the 2 GHz one on which the positioning measurements are performed – are on the same vertical line as the antenna reference point. This centimeter-level error could be ignored during the early years of the DORIS positioning, but it was taken into account when Alcatel antennas were surveyed prior to removal, during the network’ renovation phase.

Initial coordinates determination.

Prior to the launch of the first DORIS instrument on board SPOT-2, IGN published an initial set of coordinates for the DORIS network, labeled JCOD0. These coordinates were expressed either in the BTS87 realization of the BTS system (BTS: BIH Terrestrial System, the predecessor of the ITRS) or in the early realizations of the ITRF: ITRF88 or ITRF89. The reference epoch was 1984.0. This set of coordinates was later complemented as new stations were deployed after the start of the DORIS system’s operation, in the form of updates of the initial set, labeled JCOD0.n.

Such geocentric coordinates could be obtained in different ways [Boucher and Fagard 1991], from the geodetic tie between the DORIS antenna and another geodetic point in the vicinity:

- If the DORIS antenna was tied to a VLBI antenna or SLR telescope, which were generally already part of the BTS87 or ITRFnn solution, the accuracy of the resulting coordinates was better than 10 cm.

- If the DORIS antenna was tied to a Doppler Transit point, either already determined or observed simultaneously to the DORIS installation, the resulting coordinates had to be transformed from the ephemeris system (such as NSWC-9Z2, NWL-9D or WGS84) into BTS87 using a seven parameter transformation [BIH 1988]. The resulting coordinates accuracy was around one meter if precise ephemerides had been used in the computation of the Transit point, vs 2 to 10 meters with broadcast ephemerides.

- In a few cases, the DORIS antenna could only be connected to the local geodetic network, and the coordinates expressed in the national datum were transformed to BTS87 using available transformation parameters, notably those determined by the Defense Mapping Agency. Depending on the accuracy of the transformation used, the resulting  accuracy for the coordinates was between 2 and 10 meters.

# THE NETWORK DENSIFICATION: THE STAREC ERA

A new antenna model started being used as of mid-1992, instead of the original Alcatel design, whose deployment ended in September 1992 with the installation of the two Australian stations at Orroral and Yaragadee. The number of stations in the network kept on increasing until the end of 1993, when it stabilized around 50 stations, before increasing again slightly at the end of the 90’s. During this period (1994 to 1999) several stations were moved to new locations, and a few had to be upgraded either following beacons failures or damages caused to antennas by strong storms. A second generation beacon was installed at a few sites as of 199?, but was never deployed at a large scale.

# Description of the second generation equipment

The new antenna model (Figure ?), manufactured by Starec, France, offered several improvements with respect to the original Alcatel model:

- Thanks to its slimmer design, it catches the wind far less, being therefore less prone to damage by storms,

- Its phase center location is better defined (to within 1 mm, vs 5 mm for the Alcatel antennas),

- Its slimmer and more rigid design allow a more precise survey and centering to be carried out.

From its very first deployments, this antenna model was mounted on a triangular plate, linked to the underneath support by screws and nuts that allow a very fine adjustment of the antenna verticality.

The new beacon (Figure ?), called “2.0 DORIS beacon”, manufactured by SOREP, France, had the following differences with respect to the original 1.0 beacon:

- Much lighter (8 kg) and compact,

- Waterproof casing allowing its deployment in more humid environments,

- External power supply (the internal one on the first generation beacons was the cause of most failures),

- Lower power consumption (30 W vs 120 W for the 1.0 model) allowing to install it in locations where electrical power supply is limited,

- User interface only through an external computer. The beacon itself has no indication telling if it is in transmission or standby mode.

The first second generation beacon was installed at ?? in ????, and a maximum number of ?? units have been operating in the network in ????.

| Figure ? Base of the Starec antenna on a triangular plate mounted on top of a guyed lattice tower | Figure ? The DORIS 2.0 beacon and power supply |
| --- | --- |

# Starec antenna layouts

References

BIH (1988) Annual report for 1987. Observatoire de Paris.

Boucher C, Fagard H (1991) DORIS stations coordinates. Proc International DORIS orbit computation workshop, Toulouse, Oct.